# Question 6: Concept Map - Digital Image Formation

## Introduction

Digital image formation is the process of converting a real-world scene into a digital image that can be processed by a computer. Light reflected from objects passes through a camera lens and reaches an image sensor where it is converted into electrical signals. These analog signals are digitized through sampling and quantization to create a digital image. The quality of each stage directly affects image analysis, feature extraction, and Computer Vision applications.

## Concept Map

Real-World Scene
      ↓
Illumination
      ↓
Reflection
      ↓
Camera Lens
      ↓
CCD / CMOS Sensor
      ↓
Analog Signal
      ↓
Analog-to-Digital Converter
      ↓
Sampling
      ↓
Quantization
      ↓
Digital Image
      ↓
Image Enhancement
      ↓
Feature Extraction
      ↓
Object Recognition
      ↓
Decision Making

## Detailed Explanation

### Real-World Scene

Objects reflect light from natural or artificial sources. This reflected light contains information about color, texture, shape, and depth.

### Illumination

Proper illumination improves visibility and contrast. Poor lighting introduces shadows, glare, and reduced image quality.

### Camera Lens

The lens focuses incoming light onto the sensor. Aperture, focal length, and focus determine sharpness, field of view, and optical distortion.

### CCD/CMOS Sensor

The image sensor converts photons into electrical signals. CMOS sensors are widely used because of their low power consumption and fast operation, while CCD sensors offer excellent image quality.

### Analog Signal

The sensor initially generates continuous electrical signals proportional to light intensity. These signals cannot be directly processed by computers.

### Analog-to-Digital Converter (ADC)

The ADC converts analog signals into binary values through sampling and quantization.

### Sampling

Sampling divides the image into pixels. Higher sampling rates increase spatial resolution and preserve fine details, whereas low sampling causes aliasing and loss of detail.

### Quantization

Quantization assigns discrete intensity levels to pixels. Increasing the number of gray levels improves contrast and reduces quantization error.

### Digital Image

The sampled and quantized pixel values form the digital image that is stored and processed by Computer Vision algorithms.

### Image Enhancement

Noise removal, contrast enhancement, histogram equalization, and sharpening improve image quality before analysis.

### Feature Extraction

Edges, corners, textures, colors, and shapes are extracted to represent objects efficiently for recognition algorithms.

### Object Recognition and Decision Making

Machine learning and deep learning models identify objects and support decisions in applications such as autonomous driving, surveillance, healthcare, and robotics.

## Applications

Autonomous Driving

Medical Imaging

Face Recognition

Industrial Inspection

Satellite Imaging

Robotics

Remote Sensing

## Advantages

Produces computer-readable images

Improves feature extraction

Supports accurate object recognition

Foundation of Computer Vision

Enables intelligent decision making

## Conclusion

Digital image formation is the foundation of all Computer Vision systems. Careful control of sensing, sampling, quantization, and enhancement produces high-quality images that improve recognition accuracy and overall system performance.

## References

1. Gonzalez & Woods, Digital Image Processing, 4th Edition.
2. Richard Szeliski, Computer Vision: Algorithms and Applications.
3. Sonka et al., Image Processing, Analysis and Machine Vision.
4. OpenCV Documentation.